# Temperature Monitoring Study

Assistant Tool

2025

Version 1.0 D83PHT

Draft 2 - September 2025

# About Temperature Monitoring Studies

# Main Menu

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT.
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies

This guide is a quick reference and does not replace official training or protocol requirements.

# Objective

This document provides guidance to users of the Temperature Monitoring Study Application (TMS) for collecting, analyzing, and reporting temperature data.

The aim of this HELP document is to assist in proper application use, data interpretation, and report generation.

This is a supplemental aid, not a substitute for formal training on temperature studies or the use of this application.

The document is targeting trained personnel who need a quick reference when using this app.
For full training, refer to official WHO training materials, Your established SOPs or contact support (support@invgap.org).

# Preparation for conducting Temperature Monitoring Studies

Step 1: Determine the number of studies to be conducted.  It is recommended conducting two studies in two opposite seasons (Winter and Summer) to account for seasonal variations.

Step 2: Define the number of routes to be included in each of the studies.  For the purpose of accurate comparison between two studies, it is recommend to keep the number and the routes identical in both studies.

Step 3: In the <Main Menu> enter the relevant information in four mandatory fields highlighted in yellow.

Important notes:

- Specifications of the application:

  - Number of routes: 1 to 7 (at least one route required)

  - Supply chain levels: 2 to 5 (at least two levels required)

- Number of products per route: 1 to 3 (at least one vial of vaccine or other product is required to be in each route)

- Temperature Monitoring Device Requirements (TMD)

  - Any accurate digital logger recording date, time, and temperature in degree Celsius. WHO-approved devices for reliability.

  - Battery Life: Minimum 6 months.

  - Memory Capacity: At least 132,000 records.

  - Recording Interval: 30 minutes (must be pre-configured).

  - Try to use two devices per route for security, but only one dataset can be uploaded to the app.

- This application supports study length of four months (132,000 records) and not more.  If at least one of the routes of your study expected to take longer than six months, ask for support (support@invgap.org).

- Consider creating a backup copy.

Cell Color Coding System

| Cell color | meaning | Editable? |
| --- | --- | --- |
| Yellow | Mandatory fields | ✅ Yes (required) |
| White | Optional fields | ✅ Yes |
| Grey | Protected/locked cells | ❌ No |

Step 4: Save the file with a clear naming convention (e.g., "StudyName_DDMMYYYY")

Step 5: On the Main Menu, proceed to <Settings> sheet by selecting Settings from J3 and clicking on L3 or clicking on the <Settings> sheet at the bottom of your Excel window.

Major steps for conducting Temperature Monitoring Studies

Major Step 1: Preparation for Temperature Mapping Studies

- Route Planning:

  - Determine the number of routes based on logistics (geographic coverage, transit times, risk zones, etc.) – refer to WHO gridlines.

  - Select routes randomly (unbiased data) or systematically (target high-risk areas).

- Team Training:

  - Train personnel on TMS application usage (data input, report generation troubleshooting).

  - Standardize protocols for sensor handling, form completion, and incident reporting.

Output: Documented route plan, trained team, and finalized study design.

Major Step 2: TMS Application Configuration

- Complete the <Settings> sheet with:

  - Sensors’ serial numbers, route details, product samples, and developing drop-down menus.

  - Temperature thresholds (e.g., 2–8°C for vaccines) and alert parameters.

- Validate application functionality

Output: Fully configured TMS application ready for form development.

Major Step 3: Shipment and Documentation Preparation

- Form Preparation:

  - Generate forms from TMS application

- Shipment Assembly:

  - Pack sensors in real-world conditions (e.g., passive cold chain equipment).

  - Include required forms for each route separately

Output: shipment with traceable documentation ready for dispatch from the central level.

Major Step 4: Data Collection and Real-Time Monitoring

- Download copy of the completed forms from the WhatsApp group

- Update the <Monitor> sheet in TMS application:

  - Input manual observations (e.g., delays).

  - Flag discrepancies for later cross-validation with sensor data.

Output: Hybrid dataset (manual + automated) for comprehensive analysis.

Major Step 5: Sensor Data Retrieval and Upload

- Collect hard copy of the completed forms from field teams at the end of the study and keep them for reference.

- Retrieve sensors at the end of the study from each route.

- Upload recorded data to TMS – In <RecordedDat> sheet.

- Validate data completeness (e.g., no missing intervals, etc.).

Output: Consolidated dataset ready for analysis.

Major Step 6: Data Interpretation and Reporting

- Analysis:

  - Identify excursions (temperature breaches) and root causes (e.g., long stops, poor packaging, non-compliance with SOPs, equipment failure, human error).

  - Use charts, tables and reports for each route separately from TMS application.

- Reporting:

  - Summarize findings with visualizations.

  - Recommend corrective actions (e.g., route optimization, packaging upgrades, etc.).

Output: Evidence-based report with actionable insights.

Major Step 7: Comparative Analysis of Studies

- Compare results across studies to:

  - Assess seasonal impacts (summer vs. winter performance).

  - Measure improvement post-interventions (e.g., reduced temperature excursions and improve safety of the products).

- Use statistical methods (e.g., t-tests) to validate significance.

Output: Final report with long-term recommendations.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org

Note by users: …………………