# Settings

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies.

This guide is a quick reference for TMS tool users and does not replace official training or protocol requirements.

This worksheet is designed for entering study setup information. Each file corresponds to one single study containing 1 to 7 routes.

Key Rules for Data Entry

- Mandatory fields (yellow) - Must be completed before starting the study such as number of routes, name of the routes, name of levels, type of facilities, etc.  Do not miss entering the information in any of the mandatory fields.

- Optional Fields (white) - Provide additional study details if available and required.  This may include secondary parameters.

- Drop-down menus - Click the arrow ▼ to select from predefined options.  Some might be mandatory some not.

- Protected Cells (grey) - Contain formulas or texts - Cannot be modified.

Route are all color-coded.  Color codes can not be changed.

Step 1: Start completing data in this sheet:

- Specify the produce stores and transported in th supply chain (in Cell C2)

- Specify the number of routes from 1 to 7 in F1

- Specify number of existing supply chain levels from 2 to 5 in F2

- Specify the temperature lower limit alert and the higher alert limit in degree C (OC) in cells F3 and H3, respectively.  For vaccines, specify 2OC and 8OC.

- Enter level names (digital to long names) in column B, rows 7 to 12

- Enter route names in row 6 column D to J (depending on number of routes) decided to be involved in the study

- Enter the central facility name in cells C7 (they can be abbreviated name)

- Type name of the facilities for route 1 in cells D8 to D12

- Repeat item 7 for all other 6 or less routes in their corresponding columns and rows

- Specify the type of the temperature monitoring device (sensor) in cell C17 for route 1

- Repeat item 10 in cell C18 if there are two temperature monitoring devices (sensors) are in use

- Specify the temperature monitoring device serial number in cell D21 for route 1

- Repeat item 12 for all other routes in their corresponding cells.  Note that the types of the sensors are the same, but their specifications (serial numbers) must be different.

Step 2: Defining drop-down menus

Select by ticking () or define the following:

- Storage equipment used within the supply chain (column C, row 26 to 35)

- Transportation means used for moving products within the supply chain (column E, rows 26 to 35)

- Types of roads used for transportation of the products (column G, rows 26 to 35)

- Types of containers used for transportation of products (column I, rows 26 to 35)

- Types of vaccination sessions (column K, rows 26 to 29)

- Types of container used during the vaccination sessions (column K, rows 31 to 35)

If the produce in this supply chain is something different from vaccine, ignore the last two bullet points (above).

Use “Other (specify)” when the selection does not contain your choice.

The drop-down menus will automatically generate reference tables in the <Monitor> sheet (columns AE-AI, rows 4-15).  Check for accuracy.

Important notes:

- Do not modify any information in this sheet after study begins.  Changes may compromise data validity.

- Verify all entries before saving.

- Consider creating a backup copy.

Step 3: Save the file and proceed to <MasterFormsA> sheet.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org

Users’ notes: ……………………..