# Temperature Monitoring Study

Assistant Tool

2025

Version 1.0 D83PHT

Draft 2 - September 2025

Contents

Temperature Monitoring Study	1

About Temperature Monitoring Studies	2

Main Menu	2

Objective	2

Preparation for conducting Temperature Monitoring Studies	2

Settings	6

Master Forms	9

Monitor	13

Recorded Data	15

# About Temperature Monitoring Studies

# Main Menu

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT.
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies

This guide is a quick reference and does not replace official training or protocol requirements.

# Objective

This document provides guidance to users of the Temperature Monitoring Study Application (TMS) for collecting, analyzing, and reporting temperature data.

The aim of this HELP document is to assist in proper application use, data interpretation, and report generation.

This is a supplemental aid, not a substitute for formal training on temperature studies or the use of this application.

The document is targeting trained personnel who need a quick reference when using this app.
For full training, refer to official WHO training materials, Your established SOPs or contact support (support@invgap.org).

# Preparation for conducting Temperature Monitoring Studies

Step 1: Determine the number of studies to be conducted.  It is recommended conducting two studies in two opposite seasons (Winter and Summer) to account for seasonal variations.

Step 2: Define the number of routes to be included in each of the studies.  For the purpose of accurate comparison between two studies, it is recommend to keep the number and the routes identical in both studies.

Step 3: In the <Main Menu> enter the relevant information in four mandatory fields highlighted in yellow.

Important notes:

- Specifications of the application:

  - Number of routes: 1 to 7 (at least one route required)

  - Supply chain levels: 2 to 5 (at least two levels required)

- Number of products per route: 1 to 3 (at least one vial of vaccine or other product is required to be in each route)

- Temperature Monitoring Device Requirements (TMD)

  - Any accurate digital logger recording date, time, and temperature in degree Celsius. WHO-approved devices for reliability.

  - Battery Life: Minimum 6 months.

  - Memory Capacity: At least 132,000 records.

  - Recording Interval: 30 minutes (must be pre-configured).

  - Try to use two devices per route for security, but only one dataset can be uploaded to the app.

- This application supports study length of four months (132,000 records) and not more.  If at least one of the routes of your study expected to take longer than six months, ask for support (support@invgap.org).

- Consider creating a backup copy.

Cell Color Coding System

| Cell color | meaning | Editable? |
| --- | --- | --- |
| Yellow | Mandatory fields | ✅ Yes (required) |
| White | Optional fields | ✅ Yes |
| Grey | Protected/locked cells | ❌ No |

Step 4: Save the file with a clear naming convention (e.g., "StudyName_DDMMYYYY")

Step 5: On the Main Menu, proceed to <Settings> sheet by selecting Settings from J3 and clicking on L3 or clicking on the <Settings> sheet at the bottom of your Excel window.

Major steps for conducting Temperature Monitoring Studies

Major Step 1: Preparation for Temperature Mapping Studies

- Route Planning:

  - Determine the number of routes based on logistics (geographic coverage, transit times, risk zones, etc.) – refer to WHO gridlines.

  - Select routes randomly (unbiased data) or systematically (target high-risk areas).

- Team Training:

  - Train personnel on TMS application usage (data input, report generation troubleshooting).

  - Standardize protocols for sensor handling, form completion, and incident reporting.

Output: Documented route plan, trained team, and finalized study design.

Major Step 2: TMS Application Configuration

- Complete the <Settings> sheet with:

  - Sensors’ serial numbers, route details, product samples, and developing drop-down menus.

  - Temperature thresholds (e.g., 2–8°C for vaccines) and alert parameters.

- Validate application functionality

Output: Fully configured TMS application ready for form development.

Major Step 3: Shipment and Documentation Preparation

- Form Preparation:

  - Generate forms from TMS application

- Shipment Assembly:

  - Pack sensors in real-world conditions (e.g., passive cold chain equipment).

  - Include required forms for each route separately

Output: shipment with traceable documentation ready for dispatch from the central level.

Major Step 4: Data Collection and Real-Time Monitoring

- Download copy of the completed forms from the WhatsApp group

- Update the <Monitor> sheet in TMS application:

  - Input manual observations (e.g., delays).

  - Flag discrepancies for later cross-validation with sensor data.

Output: Hybrid dataset (manual + automated) for comprehensive analysis.

Major Step 5: Sensor Data Retrieval and Upload

- Collect hard copy of the completed forms from field teams at the end of the study and keep them for reference.

- Retrieve sensors at the end of the study from each route.

- Upload recorded data to TMS – In <RecordedDat> sheet.

- Validate data completeness (e.g., no missing intervals, etc.).

Output: Consolidated dataset ready for analysis.

Major Step 6: Data Interpretation and Reporting

- Analysis:

  - Identify excursions (temperature breaches) and root causes (e.g., long stops, poor packaging, non-compliance with SOPs, equipment failure, human error).

  - Use charts, tables and reports for each route separately from TMS application.

- Reporting:

  - Summarize findings with visualizations.

  - Recommend corrective actions (e.g., route optimization, packaging upgrades, etc.).

Output: Evidence-based report with actionable insights.

Major Step 7: Comparative Analysis of Studies

- Compare results across studies to:

  - Assess seasonal impacts (summer vs. winter performance).

  - Measure improvement post-interventions (e.g., reduced temperature excursions and improve safety of the products).

- Use statistical methods (e.g., t-tests) to validate significance.

Output: Final report with long-term recommendations.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org

# Settings

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies.

This guide is a quick reference for TMS tool users and does not replace official training or protocol requirements.

This worksheet is designed for entering study setup information. Each file corresponds to one single study containing 1 to 7 routes.

Key Rules for Data Entry

- Mandatory fields (yellow) - Must be completed before starting the study such as number of routes, name of the routes, name of levels, type of facilities, etc.  Do not miss entering the information in any of the mandatory fields.

- Optional Fields (white) - Provide additional study details if available and required.  This may include secondary parameters.

- Drop-down menus - Click the arrow ▼ to select from predefined options.  Some might be mandatory some not.

- Protected Cells (grey) - Contain formulas or texts - Cannot be modified.

Route are all color-coded.  Color codes can not be changed.

Step 1: Start completing data in this sheet:

- Specify the produce stores and transported in th supply chain (in Cell C2)

- Specify the number of routes from 1 to 7 in F1

- Specify number of existing supply chain levels from 2 to 5 in F2

- Specify the temperature lower limit alert and the higher alert limit in degree C (OC) in cells F3 and H3, respectively.  For vaccines, specify 2OC and 8OC.

- Enter level names (digital to long names) in column B, rows 7 to 12

- Enter route names in row 6 column D to J (depending on number of routes) decided to be involved in the study

- Enter the central facility name in cells C7 (they can be abbreviated name)

- Type name of the facilities for route 1 in cells D8 to D12

- Repeat item 7 for all other 6 or less routes in their corresponding columns and rows

- Specify the type of the temperature monitoring device (sensor) in cell C17 for route 1

- Repeat item 10 in cell C18 if there are two temperature monitoring devices (sensors) are in use

- Specify the temperature monitoring device serial number in cell D21 for route 1

- Repeat item 12 for all other routes in their corresponding cells.  Note that the types of the sensors are the same, but their specifications (serial numbers) must be different.

Step 2: Defining drop-down menus

Select by ticking () or define the following:

- Storage equipment used within the supply chain (column C, row 26 to 35)

- Transportation means used for moving products within the supply chain (column E, rows 26 to 35)

- Types of roads used for transportation of the products (column G, rows 26 to 35)

- Types of containers used for transportation of products (column I, rows 26 to 35)

- Types of vaccination sessions (column K, rows 26 to 29)

- Types of container used during the vaccination sessions (column K, rows 31 to 35)

If the produce in this supply chain is something different from vaccine, ignore the last two bullet points (above).

Use “Other (specify)” when the selection does not contain your choice.

The drop-down menus will automatically generate reference tables in the <Monitor> sheet (columns AE-AI, rows 4-15).  Check for accuracy.

Important notes:

- Do not modify any information in this sheet after study begins.  Changes may compromise data validity.

- Verify all entries before saving.

- Consider creating a backup copy.

Step 3: Save the file and proceed to <MasterFormsA> sheet.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org

# Master Forms

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT.
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies.

This guide is a quick reference for the users of TMS tool and does not replace official training or protocol requirements.

This worksheet (MasterFormsA) is totally protected. The forms are developed automatically from the data entered in <Settings> sheet.  If you desire to change anything in the forms, you should refer to <Settings> sheet. The only option in this sheet is for selecting the route number from cell E1 (drop-down menu).

Step 1: Select route number 1 from cell E1

Step 2: Print all 10 or lower number of forms  – The number of forms depends on the number of the supply chain levels specified for each route in <Settings> sheet cell F2.

  - Each form has a specific color related to the route number (column S) and

  - Each route also has a unique color refers to the form number (column T)

Users cannot change the color codes.

Each form must be color printed on a single A4 paper.   Since all routes and forms are color coded, it essential to color print the form.

Consider printing some brief instructions on the back of the forms.  This will help the field staff to perform their task in a more efficient way and reduces errors. See suggestions provided for printing on the back of the forms.

Step 3: Insert appropriate number of forms specific to each route in the corresponding  container– Check the route’s color code; ensuring:

  - The number of forms matches the supply chain levels for the route.

  - No forms from other routes are mixed in (double-check colors).

Check:

- Verify route color before inserting forms.

- Count the forms to ensure the correct quantity (as per <Settings> cell F2)

- Keep forms separated by route—especially important if handling multiple routes (e.g., seven routes).

Suggested items to be printed on the back of the forms

Print:

- a picture showing VVM stages

- the image of the temperature monitoring devices (sensors)

- a checklist how to complete the forms

- a flow chart to show where the shipment is coming from and is going to

- some useful warnings to the field staff such as “Do not touch the sensors!”, etc.

A sample of what to be printed on the back of the forms

| 1 | Inspect the content of the box. | Done |
| --- | --- | --- |
| 2 | Once your inspection of the content of the box is completed and you see no discrepancies, tick the boxes in yellow background. | Done |
| 3 | Write the date and the time you received the box | Done |
| 4 | Check the status of the VVM’s on the vial(s) of vaccines and tick the corresponding stages of VVMs (1 to 4) | Done |
| 5 | Fill the other fields on this form | Done |
| 6 | Store the box with the other vaccines. Beware that the box if contents penta vaccine that is highly sensitive to sub-zero temperatures. Therefore, it should not be stored in freezers and/or freezer rooms. | Done |
| 7 | Write your name, your position and your telephone number, and sign the form in the corresponding boxes. | Done |
| 8 | Take a photo of the completed form (use your mobile phone) and immediately send the photo of the form to the WhatsApp group “Temperature Mapping and Monitoring in Vaccine Supply Chain”. | Done |
| 9 | Keep the box with vaccines in your store for not more than 30 days, and dispatch the box with the first shipment to the designated and specified District Vaccine Store. | Done |

A sample of a checklist to be printed on the back of the forms

VVM stages                                           Image of samples of the temperature monitoring devices (sensors)

Note on VVM stages:

While the four-stage VVM classification is no longer used for field decision making and it has been replaced by a simpler binary "Use" or "Do Not Use", the four stages remain critical for temperature monitoring studies. The progressive color change allows investigators to qualitatively analyze heat exposure across different segments of the cold chain. By identifying the specific stage of change, they can pinpoint, to a large extent, where and when (the location and the time) a temperature excursion occurred.

Temperature monitoring devices (sensors)

Any accurate temperature monitoring device (sensor) can be used for these studies.  Try to use WHO tested and approved devices.  Configure the devices to measure and record temperatures in an interval of 30 minutes.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org

# Monitor

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies.

This guide is a quick reference for users of the TMS and does not replace official training or protocol requirements.

Now, the forms are prepared, temperature monitoring devices are configured and shipments are ready to leave the central level.

Follow the actionable, and structured workflow to ensure seamless execution of the temperature monitoring and data transfer process, with additional safeguards to prevent errors:

Step 1: Create  a dedicated communication group

- Develop a WhatsApp/Telegram group (choose based on field staff accessibility).

- Add the mobile phone numbers of all relevant staff involved in temperature monitoring studies (TMS).

- Train and instruct staff to post photo of completed arrival and dispatch forms in the group

Step 2: Regularly visit the group media (twice a day) and download the forms and save them in route separate folders.

Step 3: Manually transfer the data from the forms to the <Monitor> sheet.

- Type the dates and the times of arrivals and dispatches in corresponding cells in columns J, K and L (for dates), and  M and N (for time) for each segment of each route.

- Select VVM status from the drop-down menu in columns O, P and Q, depending on how many types of vaccines are included in the study (between 1 and 3 types).  If the product in the supply chain is not vaccine, ignore this step.

- Select freeze-watch status in column R.

- Transfer the ambient temperature from the forms to corresponding cells in column S (in OC).

- Type GPS coordinators in column T.

- Continue selecting “Means of transport”, “Container used for transportation of products” and “Storage equipment” in columns U, V and W, respectively.  Do not type anything in grey cells.

- In Column Z, type the distance the shipment travelled (optional) -Only from Dispatch forms.

- In column AE, type the date you detect in the copy or the picture of the form in the group media.

- In column AF, on the cell of the last row of each route, type the date you receive the sensor(s) and the hard-copy of the completed forms.

- The rest of the cells are protected except for Comments in column X.

Important:

The forms are color coded for each route. The same color codes are respected in <Monitor> sheet.  Be careful not to mix the routes data.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org

# Recorded Data

This document must be reviewed in conjunction with:
🔹 Temperature Monitoring Study Application (TMS), Version 1 – D83PHT.
🔹 Formal Training Materials
🔹 Relevant Standard Operating Procedures (SOPs) for temperature monitoring studies.

This guide is a quick reference for TMS users and does not replace official training or protocol requirements.

You may receive the recorded data from the field in two ways:

- Temperature monitoring devices (physical devices) or

- Downloaded recorded data as a soft copy  (by email or on removable disk, etc.)

In the case receiving the temperature monitoring devices, download the recorded data in an Excel file.

Transfer the recorded data (date, time and temperature) by copy/ pasting to <RecordedData> sheet fro each route separately.

Make sure not to mix routes.

It is advisable to save the files after transferring the data of each route in some other directories, in case you need them later.

Once this action is successfully completed and the recorded data are pasted in the right columns in <RecordedData> sheet, you are ready for obtaining results.

Update: 07 September 2025

Supported by wVSSM and IGA Team - 2025©

For support contact: support@invgap.org